# Email — advisor → owner of Caramba North (draft, not sent)

To: [Owner]

From: [Advisor]

Subject: Caramba North — drilling / vibration analysis for the buyer (memo attached)

Attachment: Pecos-Shallow-Drilling-Caramba-Vibration.pdf (or .docx)

[Owner],

Attached is the memo on drilling around Caramba North, written so it can go straight to the buyer. The short version: the public record supports — clearly — that the buyer's drilling-vibration concern is not borne out at this site.

The spine is location, and it is strong. Counting only genuine new wells (completion ≥ spud — recompletion re-stamps excluded): no well of any kind has been spudded within two miles of the tract in over a decade; no new-drill well within five miles; only three new-drill wells within ten miles across all of 2020–2025 (nearest ≈ 6.9 miles; all three deep ≥9,200 ft, spud 2020 and 2025). Pecos has just ≈116 genuine new wells county-wide since 2020 — about one-tenth of the comparable-county average (≈1,148, across Reeves, Midland, Martin, Reagan, Howard, and Loving) and the least of the seven. The wells within two miles are all decades-old legacy completions (drilled 1950s–2002) — mostly plugged shallow verticals plus a handful of long-completed deep wells.

Reinforcing that — and this is the key refinement: the large "shallow" footprint in Pecos is recompletions, not new drilling. About 45% of Pecos permits since 2020 are recompletions (rework of existing wellbores — no new hole), and 96% of every recompletion is one operator, Kinder Morgan Production, reworking existing CO₂-flood fields. Genuine new drilling (~478 permits) is ~97% deep horizontal (Diamondback, XTO, Continental, Gordy) — and, as above, essentially none of it is near the site.

A few things worth your knowing:

This is a correction I made after a good catch on the data, and it makes the memo stronger, not weaker. An earlier draft said "~50% of Pecos drilling is shallow/vertical." On checking the permit `filing_purpose` field, roughly half of that "shallow" activity is recompletions, not new wells — so the honest, cleaner statement is that the shallow footprint is rework of existing wells, while genuine new drilling is deep and remote. The proximity conclusion didn't weaken; restricting to new drills it got tighter (zero within five miles).

It also resolves an internal tension. The earlier "~50% shallow" sat awkwardly next to "permits are ~87% horizontal." With recompletions separated out, both reconcile: new drilling is ~97% deep; the shallow signal was rework. Nothing in the memo now contradicts the RRC's own data.

Production cross-reference (Finding 9) — re-run on a strict marginal threshold, per your direction. Two corrections sit underneath the current numbers, both following good catches on the data. First, the earlier "none producing within ten miles" was an artifact of a weak wellbore→lease name match that only covered ~31% of wells; I rebuilt the join by API number through RRC's authoritative API-to-lease crosswalk (≈99.6% coverage). Second, the prior 150 bbl/d oil cutoff for "marginal" was too loose — a 100 bbl/d well is not really marginal — so per your direction the threshold is now gas ≤ 125 Mcf/d AND oil ≤ 25 bbl/d, a strict marginal/end-of-life rate. Under the corrected join and the strict threshold: within ten miles, 241 of 291 non-plugged new-drill wells (~83%) are marginal or end-of-life, and the 50 still producing above that threshold are all decades-old completions — about 15 legacy deep-gas wells (1965–1978, ≈17,000–22,800 ft); a cluster of ~31 low-rate (~37.5 bbl/d) vertical conventional oil wells at ≈3,150–3,440 ft (spud 1986–2012, mostly 2007–2012 — a unitized stripper-grade pumping lease); 3 truly shallow (<3,000 ft) oil wells at 2,824–2,945 ft (2004–2011 spud); and the single 2020 deep-horizontal new-drill at 9.37 mi out (Finding 4). Within two miles, 3 of 5 non-plugged are marginal and the 2 still producing are a 1975, ≈22,100-ft gas well at 125.7 Mcf/d and a 2008, ≈3,275-ft vertical oil well at 37.5 bbl/d. County-wide marginal/EoL is ≈12,490 (still producing ≈10,060). Every still-producing well near the site is a decades-old completion — not an active drilling or hydraulic-fracturing operation — so the vibration thesis holds: drilling/fracking activity is absent. The map (lrp-tx-gis.netlify.app) was rebuilt with the same threshold so the buyer's technical team will see the same figures. The buyer copy carries the corrected numbers without the methods commentary, per your instruction.

Methods/limitations remain out of the buyer copy, per your instruction. For your awareness only: this is a drilling-record analysis, not a geotechnical vibration study. If the buyer wants engineering certainty, a short vibration assessment is the clean next step; this data underpins it.

Net: a location-first package, now built on a clean new-drill-vs-recompletion distinction that holds up under scrutiny. Happy to walk through it or adjust emphasis before it goes out.

[Advisor]

Notes (delete before sending): fill in [Owner] / [Advisor]; attach `Pecos-Shallow-Drilling-Caramba-Vibration.pdf` (or .docx). I have not sent anything. The buyer-facing version is `buyer-email.md`.